ECE 6360 Parallel and Heterogeneous Computing

Homework 1: Instruction-Level Parallelism

Problem 1. Write instruction-level pseudocode that uses a Taylor series to calculate ex and provide a dependency graph for the loop.

Dependency Graph:

float exp(float x, int n) {

}

Problem 2. The following function calculates the l2 norm (Euclidean norm) of an n-dimensional vector. Assume that each numbered line represents an operating taking a single clock cycle. The first loop (line 02) is completely independent across all n values of x and can be parallelized. The others must be run in serial. What percentage of this program is parallel? According to Amdahl’s law, what speedup can we expect if both n and the number of processors are large?

float l2(float* x, int n) {

01   int i = 0;

while(i < n) {

02     x[i] = x[i] * x[i];

}

03   i = 0;

04   float sum_sq = 0;

while(i < n) {

05     sum_sq = sum_sq + x[i]

}

06   return sqrt(sum);

}